A SIMPLE GUIDE TO THE SCRIPTS

Running the Following Robot

What each script does, and exactly which one to run to start following

⭐ The short answer — to start real-time following: Make sure the robot is switched on and connected. Then, in a terminal on the VM, run this ONE command: bash /mnt/host_project/scripts/06-follow-real.sh That single script starts the robot's motors and camera, then the person-detector and the decision-maker on the VM. The robot then drives toward the nearest person.

You do the four setup steps once; after that, one script starts everything.

# 1.  All the scripts at a glance

There are six scripts. The first four are a one-time setup you do once when preparing the machine. The last two are the ones you actually run to start the robot — one for simulation, one for the real robot.

| Script | When you run it | What it does |
| --- | --- | --- |
| 00-bootstrap-vm.sh | Once, first | Prepares the virtual machine (lets scripts run without password prompts). |
| 02-install-ros.sh | Once | Installs ROS 2, the TurtleBot3 packages, and the Gazebo simulator. |
| 03-install-yolo.sh | Once | Installs the YOLO person-detector and downloads its model file. |
| 04-build-workspace.sh | Once (and after code edits) | Builds this project's code so ROS 2 can run it. |
| 05-run-demo.sh | Every time (SIMULATION) | Runs the whole demo in the Gazebo pretend world. |
| 06-follow-real.sh | Every time (REAL ROBOT) ⭐ | Starts the real robot following a person. THIS is the follow script. |

💡 Rule of thumb: numbers 00–04 are 'install & build' (done once). Numbers 05 and 06 are 'go' — 05 for the simulation, 06 for the real robot.

# 2.  First-time setup (run once)

These four prepare everything. You run them in order the first time only. Each prints coloured progress and is safe to re-run if something is interrupted.

## 00-bootstrap-vm.sh — prepare the VM

A tiny helper that lets the other scripts run smoothly without stopping to ask for a password, and makes the project folder easy to reach inside the VM.

## 02-install-ros.sh — install ROS 2 + Gazebo

Installs the robot 'operating system' (ROS 2 Humble), the TurtleBot3 packages, and the Gazebo simulator. The key lines it runs:

sudo apt install -y ros-humble-desktop ros-dev-tools ... # ROS 2 itself sudo apt install -y ros-humble-turtlebot3 ros-humble-turtlebot3-gazebo ... # robot + simulator sudo apt install -y tmux # runs several panes at once

→ In plain words: it downloads and installs all the robot software so the computer 'speaks robot'.

## 03-install-yolo.sh — install the detector

Installs the YOLO detector (the software that recognises people) and downloads its small 'brain' file, plus the py_trees library used for the decision-making.

python3 -m pip install --user torch torchvision # the neural-network engine (CPU version) python3 -m pip install --user ultralytics # YOLOv8 person detector # ... then downloads the model file to ~/models/yolov8n.pt

→ In plain words: it installs the 'spotter' that finds people in the camera picture.

## 04-build-workspace.sh — build the project

Compiles this project's own code (the detector node, the behaviour tree, the messages) so ROS 2 can run it. You run this once, and again any time the code is changed.

cd ~/ros2_ws && colcon build --symlink-install

→ In plain words: it turns the project's source code into something the robot can actually run.

# 3.  Running the SIMULATION — 05-run-demo.sh

This is for testing without any hardware. It opens three panels side-by-side (using a tool called tmux) and starts the three parts of the system together.

SESSION="tb3-follower" # the settings every panel needs (find ROS, the project, the robot model, Gazebo) SETUP='export DISPLAY=:0 && source /opt/ros/humble/setup.bash && \ source ~/ros2_ws/install/setup.bash && export TURTLEBOT3_MODEL=waffle_pi && ...' # Panel 0: the Gazebo pretend world + the robot tmux send-keys "$SETUP && ros2 launch tb3_follower_bringup sim.launch.py" # Panel 1: the YOLO person-detector (waits 8 s for the world to load first) tmux send-keys "$SETUP && sleep 8 && ros2 launch tb3_follower_bringup perception.launch.py" # Panel 2: the behaviour-tree decision-maker (waits 10 s) tmux send-keys "$SETUP && sleep 10 && ros2 launch tb3_follower_bringup behavior.launch.py"

→ In plain words: it opens the pretend world, the person-detector, and the decision-maker in three panels, one after another, so the robot follows a walking person in simulation.

Run it, watch it, stop it:

bash /mnt/host_project/scripts/05-run-demo.sh # start tmux kill-session -t tb3-follower # stop

# 4.  Running the REAL robot — 06-follow-real.sh  ⭐

This is the main script — the one you run to make the real robot follow a person. It is clever because it starts things in the right place automatically: the robot's own parts on the robot's computer (the Raspberry Pi), and the 'thinking' parts on the VM.

The seven things 06-follow-real.sh does — some on the Pi, some on the VM.

## Reading the script, block by block

Block 1 — settings. Where the robot is, how to log in, and which network number to use.

PI_HOST="192.168.137.105" # the robot's address PI_USER="ubuntu"; PI_PASS="turtlebot" DOMAIN="30" # the shared ROS network number (VM and robot must match) TB3_MODEL="waffle_pi"

→ In plain words: this is just the address book — who to call and on which channel.

Block 2 — get ready on the VM. Load ROS 2, load the project, and join the robot's network.

source /opt/ros/humble/setup.bash source ~/ros2_ws/install/setup.bash export ROS_DOMAIN_ID=30 ROS_LOCALHOST_ONLY=0 # then check we can actually reach the robot, or stop with a clear error: $SSH 'echo ok' ¦¦ die "Cannot reach the Pi — check the network."

→ In plain words: it gets the VM ready and makes sure the robot is reachable before going further.

Block 3 — start the robot's drivers ON THE PI (only if not already running). This turns on the motors, the LiDAR and odometry.

$SSH 'bash -s' <<REMOTE ... export ROS_DOMAIN_ID=30 TURTLEBOT3_MODEL=waffle_pi LDS_MODEL=LDS-02 if pgrep -f robot.launch >/dev/null; then echo "already running" else ros2 launch turtlebot3_bringup robot.launch.py & fi REMOTE

→ In plain words: it logs into the robot and switches on its motors and laser.

Block 4 — start the camera ON THE PI (again, only if needed), at a small size so it streams smoothly over Wi-Fi.

$SSH 'bash -s' <<REMOTE ros2 launch realsense2_camera rs_launch.py \ enable_depth:=false rgb_camera.color_profile:=424x240x15 & REMOTE

→ In plain words: it switches on the robot's camera.

Block 5 — wait for the sensors. The script pauses until the laser scan and the camera pictures are actually arriving at the VM, so the follower doesn't start blind.

for i in $(seq 1 40); do ros2 topic list ¦ grep -q "/scan" && ros2 topic list ¦ grep -q "$COLOR_TOPIC" && break sleep 2 done

→ In plain words: it waits until it can see the robot's laser and camera data before continuing.

Block 6 — start the 'brain' on the VM. It opens a tmux session called tb3-follow and launches the person-detector and the behaviour tree.

tmux new-session -d -s tb3-follow # panel: the YOLO person-detector, reading the robot's camera tmux send-keys "... ros2 run tb3_follower_perception person_detector_node ..." # panel: the behaviour tree, which sends drive commands (/cmd_vel) to the robot tmux send-keys "... ros2 launch tb3_follower_bringup behavior.launch.py"

→ In plain words: it starts the person-detector and the decision-maker; from here the robot follows the nearest person.

💡 It's safe to re-run: if parts are already running (the robot drivers, the camera), the script notices and skips them — so running it twice does no harm.

# 5.  The exact run order for real-time following

Putting it all together, here is the complete sequence to make the real robot follow you:

One time only: run 00-bootstrap-vm.sh, then 02-install-ros.sh, then 03-install-yolo.sh, then 04-build-workspace.sh (in that order). You never repeat these unless you rebuild the machine.

Switch the robot on and wait a minute or two for it to start up and join the Wi-Fi hotspot.

Check the connection — from the VM you should be able to reach the robot (the follow script checks this for you and stops with a clear message if not).

Start following — in a VM terminal run the one command:

bash /mnt/host_project/scripts/06-follow-real.sh

Stand a couple of metres in front of the robot — it detects you and drives toward you, keeping about a 1-metre gap, turning to face you, and steering around obstacles.

So, which script starts real-time following? 06-follow-real.sh Everything else is either one-time setup (00–04) or the simulation (05). For the real robot, this is the only script you run.

# 6.  Watching and stopping

## Watch what's happening

tmux attach -t tb3-follow # see the detector + behaviour panels (leave with Ctrl+B then D) ros2 topic echo /cmd_vel # the drive commands going to the robot ros2 topic echo /follower/state # the robot's current action (approaching / holding / avoiding ...)

## Stop it

# stop the follower on the VM: tmux kill-session -t tb3-follow # stop the robot's drivers + camera (from the VM): sshpass -p turtlebot ssh ubuntu@192.168.137.105 \ "pkill -f robot.launch; pkill -f realsense2_camera_node"

💡 Safety: the real robot moves. Give it clear floor space, and keep the stop command (or the robot's power switch) within reach.

# 7.  Quick reference card

| I want to... | Run this |
| --- | --- |
| Set up the machine (once) | 00 → 02 → 03 → 04 (in order) |
| Run the SIMULATION | bash scripts/05-run-demo.sh |
| Start the REAL robot following ⭐ | bash scripts/06-follow-real.sh |
| Watch the panels | tmux attach -t tb3-follow |
| Stop the real follower | tmux kill-session -t tb3-follow |

— End of scripts guide —